.mt 4

.mb 7

.cw 12

.op

.ul on

.lh 8

.lq off

.ps off

.po 8

.sr 3

TO:       Bill Henry

Gary Musgrave

FROM:     A.B. White

SUBJECT:  ST-8000A KEYPAD OPERATION

DATE:     December 23, 1990

.fo ABW 12/23/90 keypad2.doc

.he ST-8000A KEYPAD OPERATION                                  Page #

The ST-8000A keypad provides complete control over the  operation

of  the ST-8000A modem.  It is used to set the MARK and SPACE  or

CENTER and SHIFT frequencies for the demodulator (channel 1)  and

the modulator (channel 2) as well as set various other parameters

for both channels and activate built in test routines.

Whenever parameters are entered, the ENTER key indicator  flashes

to  signal  the change.  To save this new parameter in  the  non-

volatile memory, the user need only press the ENTER key.  At  any

time  before  ENTER is pressed, the CLEAR key may be  pressed  to

ignore the new setting and return the ST-8000A to the  previously

saved parameters.

In  the  following sections, the function of each of  the  keypad

function  keys is summarized.  The keys are presented  in  keypad

order starting with the left hand column.

1.  NUMBER KEYS

The  number  keys,  0  to  9 and  0.5,  are  used  to  enter  new

frequencies, baud rates, and regeneration word length, and select

internal BIT tests.  The number keys are only active after one of

the  following function keys has been pressed: MARK, SPACE,  BAUD

RATE, or SYNC.  When entering a number, the corresponding display

window  shows  the  new setting, but the  new  parameter  is  not

activated  until the ENTER key is pressed.  If the CLEAR  key  is

pressed  before  ENTER, the modem will return to  the  previously

stored settings and will ignore the new setting.

2.  ENTER KEY

The ENTER key stores the current modem parameters in non-volatile

memory and turns OFF the flashing ENTER indicator.  Pressing  any

parameter key causes the ENTER indicator to flash.  If the  CLEAR

key is pressed before ENTER, the modem returns to the  previously

stored  parameters.   Note: the REMOTE key does not  require  the

ENTER key.

3.  M/S  CENT KEY

The  M/S  key  determines the front panel display  mode  and  the

parameter entry mode.  Pressing the M/S key turns ON the MARK and

SPACE  indicators and displays the MARK and SPACE  filter  center

frequencies  when  channel 1 is selected, or  the  transmit  tone

frequencies  when channel 2 is selected.  In this mode, the  MARK and  SPACE keys set the MARK and SPACE frequencies directly  for

the selected channel.

Pressing 2ND then M/S changes the front panel display to show the

CENTER and SHIFT frequencies for either channel 1 or 2.  In  this

mode,  the  indicators next to the CENTER and  SHIFT  labels  are

turned ON.  In addition, the MARK and SPACE keys load the  CENTER

and SHIFT frequencies, respectively, for the selected channel.

To  save the new display and parameter entry mode, the ENTER  key

must be pressed.

4.  BIT KEY

The  BIT routines are activated by pressing 2ND the BIT  followed

by  ENTER or a number and ENTER.  If ENTER alone is pressed,  the

complete BIT procedure is performed.  The table below  summarizes

the  available test features and the corresponding  BAUD  display

message.

KEY       DESCRIPTION                        BAUD DISPLAY

-----     ---------------------------------  ------------

ENTER     Automatic BIT procedure             -

1         Automatic BIT procedure             -

2         Constant MARK                       "_---"

3         Constant SPACE                      "-___"

4         Alternating MARK/SPACE at TX BAUD   "Alt "

7         Internal Loopback at 0 dBm          "LP 1"

8         Internal Loopback at -20 dBm        "LP 2"

9         Internal Loopback at -40 dBm        "LP 3"

5.  2ND KEY

The 2ND key accesses the second level functions on certain  keys.

The  ST-8000A  has second level functions only on  the  following

keys:  M/S CENT, BIT, MARK ONLY, SPACE ONLY, AMH HOLD, NORM  REV,

and SYNCH REGEN.  Pressing any key other than those listed  above

returns the keypad to first level functions.

.pa

6.  FSK KEY

The  FSK  key is used with MARK ONLY and SPACE ONLY  to  set  the

demodulator  discriminator mode.  Pressing FSK turns ON  the  FSK

indicator  and selects the MARK/SPACE discriminator  mode.   This

key is functional whether channel 1 or channel 2 is selected.

KEYS       ACTION

-----      -------------------------------------------------

FSK        Select demodulator MARK/SPACE mode.

Turns ON the FSK indicator.

To save the new demodulator mode, the ENTER key must be pressed.

7.  MARK ONLY KEY

The MARK ONLY key has both a first and a second level function.

7.1  MARK (first level):

Press  MARK alone to enter the MARK filter frequency (channel  1)

or the MARK transmit tone (channel 2) when the MARK/SPACE display

mode   is  active.   The  following  example  assumes  that   the

demodulator, channel 1, is selected.

KEYS       ACTION

-----      -------------------------------------------------

MARK       Blank the MARK display window, ENTER turns ON

2125       "2125.0" shows in the MARK window

ENTER      "2125.0" flashes, ENTER turns OFF

The MARK filter is set to 2125.0 Hz

When the display mode is set for CENTER/SHIFT, the MARK key  sets

a new CENTER frequency for the demodulator filters, channel 1, or

the  transmit  tones, channel 2.  The following  example  assumes

that the demodulator, channel 1, is selected.

KEYS       ACTION

-----      -------------------------------------------------

MARK       Blank the CENTER display window, ENTER turns ON

2210       "2210.0" shows in the CENTER window

0.5        "2210.5" shows in the CENTER window

0.5        "2210.0" shows in the CENTER window

ENTER      "2210.0" flashes, ENTER turns OFF

The demodulator center frequency is set to 2210 Hz

Note:   When the center changes, the previous shift frequency  is

maintained.  If previous MARK and SPACE were 1000.0 and 1200.0 Hz

(Shift  = 200.0 Hz), setting center to 2210 Hz will maintain  200

Hz shift and change MARK to 2110.0 Hz and SPACE to 2310.0 Hz.

To save the new center frequency, the ENTER key must be pressed.

7.2  MARK ONLY (2nd level):

Pressing 2ND then MARK enables the MARK ONLY demodulator mode and

turns  ON  the MK ONLY indicator.  If the FSK indicator  was  ON,

that indicator turns OFF.  If the SP ONLY indicator was on,  that

indicator turns OFF.

KEYS       ACTION

-----      -------------------------------------------------

2nd        Enable second level function

MARK       Select demodulator MARK ONLY mode.

Turn ON the MK ONLY indicator.

To save the new demodulator mode, the ENTER key must be pressed.

8.  SPACE ONLY KEY

The SPACE ONLY key has both a first and a second level function.

8.1  SPACE (first level):

Press SPACE alone to enter the SPACE filter frequency (channel 1)

or  the  SPACE  transmit tone (channel  2)  when  the  MARK/SPACE

display  mode is active.  The following example assumes that  the

demodulator, channel 1, is selected.

KEYS       ACTION

-----      -------------------------------------------------

SPACE       Blank the SPACE display window, ENTER turns ON

2295       "2295.0" shows in the SPACE window

ENTER      "2295.0" flashes, ENTER turns OFF

The SPACE filter is set to 2295.0 Hz

When the display mode is set for CENTER/SHIFT, the SPACE key sets

a new SHIFT frequency for the demodulator filters, channel 1,  or

the  transmit  tones, channel 2.  The following  example  assumes

that the demodulator, channel 1, is selected.

KEYS       ACTION

-----      -------------------------------------------------

SPACE      Blank the SHIFT display window, ENTER turns ON

200        " 200." shows in the SHIFT window

ENTER      " 200." flashes, ENTER turns OFF

The demodulator shift frequency is set to 200 Hz.

Note:   When the shift changes, the previous center frequency  is

maintained.   If previous center and shift were 2210.0 and  170.0

Hz,  setting  shift  to 200 Hz will retain  the  2210  Hz  center

frequency and change MARK to 2110.0 Hz and SPACE to 2310.0 Hz.

To save the new shift frequency, the ENTER key must be pressed.

8.2  SPACE ONLY (2nd level):

Pressing 2ND then SPACE enables the SPACE ONLY demodulator mode and

turns  ON  the SP ONLY indicator.  If the FSK indicator  was  ON,

that indicator turns OFF.  If the MK ONLY indicator was on,  that

indicator turns OFF.

KEYS       ACTION

-----      -------------------------------------------------

2nd        Enable second level function

SPACE      Select demodulator SPACE ONLY mode.

Turn ON the SP ONLY indicator.

To save the new demodulator mode, the ENTER key must be pressed.

9.  DIV KEY

The DIV key toggles the demodulator diversity option ON and  OFF.

When ON, the DIV indicator turns ON.

KEYS       ACTION

-----      -------------------------------------------------

DIV        Turn DIVERSITY option ON; turn ON DIV indicator

DIV        Turn DIVERSITY option OFF; turn OFF DIV indicator

To save the new demodulator mode, the ENTER key must be pressed.

10.  MUTE KEY

The MUTE key toggles the modulator mute feature ON and OFF.  This

feature  operates  correctly  only when  the  internal  modulator

jumper is set for MUTE.  When the mute option is ON and  transmit

data stops, the transmit tones are turned OFF with the PTT output

after  the keyline delay expires.  The MUTE indicator is ON  when

this  feature is enabled.  When the mute option is OFF,  the  PTT

output and the transmit tones are always ON.

KEYS       ACTION

-----      -------------------------------------------------

MUTE       Turn MUTE option ON; turn ON the MUTE indicator

MUTE       Turn MUTE option OFF; turn OFF the MUTE indicator

To save the new mute setting, the ENTER key must be pressed.

.pa

11.  REMOTE KEY

The REMOTE key selects local and remote control of the  ST-8000A.

When  REMOTE  is enabled, the REMOTE indicator turns ON  and  all

parameter control is via the rear panel remote control port.  The

REMOTE LED is ON whenever Remote control is active.  When  remote

control mode is enabled, all keypad keys are disabled except  for

the REMOTE key.

Keypad or local control is restored by pressing the REMOTE key.

KEYS       ACTION

-----      -------------------------------------------------

REMOTE     Enable REMOTE control; turn ON REMOTE indicator

REMOTE     Enable LOCAL control; turn OFF REMOTE indicator

The  local or remote control status is saved in the  non-volatile

memory immediately; no ENTER key press is required.

12.  AMH HOLD KEY

The AMH HOLD key has both a first and a second level function.

12.1  AMH (first level):

The  AMH key toggles the Automatic Mark-Hold receive  feature  ON

and OFF.  When AMH is ON, the AMH indicator is turned ON.

KEYS       ACTION

-----      -------------------------------------------------

AMH        Turn AMH option ON; turn ON the AMH indicator

AMH        Turn AMH option OFF; turn OFF the AMH indicator

To save the new AMH setting, the ENTER key must be pressed.

12.2  HOLD (2nd level):

Pressing  2ND  then HOLD toggles the MARK hold  feature  for  the

selected  channel.  When channel 1 is selected,  the  demodulator

output  is  held  in  the MARK  condition.   When  channel  2  is

selected,  the  transmit tone output is held in the  MARK  state.

The HOLD indicator on the front panel shows the current state for

the  selected  channel.  The following example assumes  that  the

transmit channel 2 is selected.

KEYS       ACTION

-----      -------------------------------------------------

2ND        Enable second level function

HOLD       Turn HOLD option ON; turn ON the HOLD indicator

2ND        Enable second level function

HOLD       Turn HOLD option OFF; turn OFF the HOLD indicator

To save the new HOLD setting, the ENTER key must be pressed.

13.  NORM REV KEY

The NORM REV key has both a first and second level function.

13.1  NORM (first level):

The  NORM  key sets the selected channel polarity to  NORMAL  and

turns OFF the REV indicator.

KEYS       ACTION

-----      -------------------------------------------------

NORM       Enable NORMAL polarity; turn OFF REV indicator

To save the new polarity setting, the ENTER key must be pressed.

13.2  REV (2nd level):

Pressing  2ND  then  REV sets the selected  channel  polarity  to

REVERSE and turns ON the REV indicator.

KEYS       ACTION

-----      -------------------------------------------------

2ND        Enable second level function

REV        Enable REVERSE polarity; turn ON REV indicator

To save the new polarity setting, the ENTER key must be pressed.

14.  SYNCH REGEN KEY

The SYNCH REGEN key has both a first and second level function.

14.1  SYNCH (first level)

The  SYNCH key toggles the clocked synchronous data mode  ON  and

OFF.  When ON, the SYNC indicator turns ON and the modem  receive

data  outputs are internally re-clocked with a recovered  mid-bit

clock.   When  OFF, the SYNC indicator turns OFF  and  the  modem

receive  data outputs are directly connected to  the  demodulator

output.

.pa

When  the SYNC indicator turns OFF, the bit display  prompts  for

the  receive data regeneration word length.  The current  setting

is  shown,  and  it may be set to 5, 6, 7, or  8  bits.   If  the

current setting is correct, ENTER only is pressed.

KEYS       ACTION

-----      -------------------------------------------------

SYNCH      Enable SYNC mode; turn ON the SYNC indicator

If REGEN mode was ON, it is turned OFF

SYNCH      Disable SYNC mode; turn OFF the SYNC indicator

"L=5 " is displayed in the BAUD window.

8          "L=8 " is displayed in the BAUD window.

ENTER      Flash the BAUD window and restore BAUD RATE

The new configuration is saved immediately.

13.2  REGEN (2nd level)

Pressing  2ND  then REGEN toggles the receive  data  regeneration

mode ON and OFF.  When ON, the REGEN indicator turns ON, the SYNC

indicator  turns  OFF,  and data  characters  received  from  the

demodulator are regenerated for the receive data outputs.

KEYS       ACTION

-----      -------------------------------------------------

2ND        Enable second level function

REGEN      Enable REGENERATION; turn ON the REGEN indicator

If SYNCH mode was ON, it is turned OFF.

2ND        Enable second level function

REGEN      Disable REGENERATION; turn OFF the REGEN indicator

To save the regeneration mode, the ENTER key must be pressed.

15.  CHAN KEY

The CHAN key alternately selects channel 1 and channel 2 for  the

front panel display and parameter entry.

KEYS       ACTION

-----      -------------------------------------------------

CHAN       Display channel 1, the demodulator

CHAN       Display channel 2, the modulator

To save the new channel selection, the ENTER key must be pressed.

.pa

16.  BAUD RATE KEY

Press BAUD RATE to enter the baud rate for the selected  channel.

If  channel 1 is selected, the BAUD RATE sets the  bandwidth  for

the receive filters, the regeneration data rate, and the  receive

data mid-bit clock center frequency.  When channel 2 is selected,

the BAUD RATE loads the new synchronous transmit clock frequency.

KEYS       ACTION

---------  -------------------------------------------------

BAUD RATE  Blank the BAUD display window, ENTER turns ON

110        " 110." shows in the BAUD window

ENTER      " 110." flashes, ENTER turns OFF

The BAUD RATE is set to 110 bits per second.

17.  CLEAR KEY

The  CLEAR key restores the last saved modem  parameter  settings

whenever  the ENTER key indicator is flashing.  This key  may  be

used  to restore the old MARK, SPACE, and BAUD RATE  settings  if

CLEAR is pressed before ENTER as a new value is entered.