.rm 65

.cw 12

.ul on

.op

.mt 4

.mb 7

.lh 8

.lq off

.ps off

.po 8

.sr 3

.cw 10

4.4.3  Front Panel Controller

.fo ABW 9-16-90 ch4_fp.doc

.he CHAPTER 4                Theory of Operation                            Page #

The  ST-8000A uses an integrated keyboard decoder and display controller  (U8)

shown in Figure A1819 to handle the front panel keypad input and seven segment

display  outputs.   This  device  debounces  keypad  entries  to  prevent  the

inadvertent  entry  of  multiple keys, and buffers the display  data  for  the

multiplexed front panel numerical displays and indicators.

The  keypad/display  interface  controller (U8) is  an  8  bit  microprocessor

peripheral device that connects directly to the control board data bus (DB0 to

DB7), the read (RD_L) and write (WR_L) signals, address bit 0 (A0), the  front

panel  enable (FP_L), and system reset (RESET).  The microprocessor reads  and

writes  the control and data registers in this device to input keypad  entries

and load the display buffer output registers.  An internal counter divides the

system clock (PCLK) input to set the keypad and display scanning rate.

The keypad and the front panel display are scanned through the binary  encoded

scan  line  outputs (SL0 to SL3) in Figure A1819 and 3-to-8 line  decoders  in

Figures  A1820 and A1821.  Keypad entries are detected as a TTL low on one  of

the row inputs (RL0 to RL3).

The seven segment display segment drivers are connected to the segment outputs

(U8-24 to U8-31).  Data on this output bus is inverted; to turn a segment  ON,

the corresponding output is held at a TTL low level when the digit is scanned.

Because the segment outputs do not have sufficient drive capability,  discrete

PNP  switching transistors (Q17 to Q24) in Figure A1819 buffer  these  outputs

for  all of the display segments.  The display segments anodes of  the  common

cathode  seven segment display digits are all driven in parallel, as shown  in

Figure A1822, and one scanned digit is selected with the digit driver  circuit

in Figure A1821.

4.4.3  Keypad

The front panel keypad is a matrix of single pole momentary switches  arranged

in three rows of 9 switches.  The low three bits of the scan line outputs (SL0

to SL2) drive a 3-to-8 line decoder (U7) to select one of 8 keypad columns  by

forcing  that column select line low.  If any of the keypad switches  in  that

column  are  closed, one of the row lines (RL0 to RL3) will be forced  low  to

signal the keypad/display controller (U8) that a key has been pressed.

Note  that  the keypad is arranged as 4 rows of 8 keys and 1 row  of  3  keys.

This  keypad/display  controller (U8) has a maximum capacity of 8  rows  by  8

columns, so it cannot scan the 9 columns in a simple matrix fashion.  However,

by  connecting three of the column drivers to the last column, and  connecting

that  column to a fourth row line (RL3), the entire matrix of switches may  be

scanned.

When a keypress is detected, the keypad/display controller (U8) stores the row

and  column locations into a key buffer and interrupts the  microprocessor  by

forcing the front panel interrupt line (KBINT) high.

ST-8000A  firmware  determines the function of all front  panel  keypad  keys;

there are no hardwired switch functions.

4.4.3  Numerical Displays

The  front panel assembly has 15 common cathode seven segment  display  digits

and two additional LEDs driven in a multiplexed fashion by the  keypad/display

controller  (U8).  The scan line outputs (SL0 to SL3) select one of  16  digit

drivers using two 3-to-8 line decoders shown in Figure A1821.  For each of the

16  scan line selections, a single common cathode digit driver  transistor  is

enabled  through an inverting buffer.  As each digit is selected in turn,  the

segments  are  turned  ON  that  correspond  to  the  data  presented  on  the

keypad/display controller data output lines in Figure A1819.

The  keypad/display  controller (U8) is configured to scan  the  entire  front

panel display every 20 ms or 50 times per second to avoid visual flicker.   In

addition,  the keypad/display controller blanks the display as the scan  lines

change so that display digit contrast is improved.

Figure  A1822  illustrates  the assignment of the digits to  the  front  panel

locations.  Five digits are assigned to the MARK and SPACE frequency  windows,

four  digits  to the BAUD window, and a single digit to the  CHANNEL  display.

The  two  LED's  mirror  the channel selection, but they  are  scanned  as  an

independent digit during scan time slot 15.

The digit assignments for the front panel are summarized below:

DISPLAY   DIGIT SCAN NUMBER

--------  -----------------

MARK      4   3   2   1   0

SPACE     9   8   7   6   5

BAUD         13  12  11  10

CHANNEL                  14

LED                      15

Each seven segment display digit has segments assigned as shown below:

BIT

SEGMENT   7  6  5  4  3  2  1  0

-------   -  -  -  -  -  -  -  -                 a

SEG dp    X                                    -----

SEG g        X                              f |     | b

SEG f           X                             |  g  |

SEG e              X                           -----

SEG d                 X                     e |     | c

SEG c                    X                    |     |

SEG b                       X                  -----   o dp

SEG a                          X                 d

4.4.3  LED Indicators

In  addition  to  the multiplexed front panel display  digits,  there  are  16

latched front panel LED indicators illustrated in Figure A1823.  The two octal

latches  (U1,  U2) are connected to the control board data bus (DB0  to  DB7).

The  signals to load the LED latches come directly from an I/O output  address decoder  on  the  control  board (LED0_L, LED1_L).   The  LED  latch  data  is

inverted; a TTL 0 turns the corresponding LED ON.

The LED data bus bit assignments for the front panel are summarized below:

BIT

LED0_L (U1)  7  6  5  4  3  2  1  0

-----------  -  -  -  -  -  -  -  -

MARK         X

CENTER          X

SPACE              X

SHIFT                 X

SYNC                     X

FSK                         X

MARK ONLY                      X

SPACE ONLY                        X

BIT

LED1_L (U2)  7  6  5  4  3  2  1  0

-----------  -  -  -  -  -  -  -  -

REGEN        X

AMH             X

HOLD               X

REV                   X

ENTER                    X

DIV                         X

MUTE                           X

REMOTE                            X